# GolfCam V3

Complete setup, REST firmware, Android deployment, operation and troubleshooting

Current architecture: Android app -> REST API -> two Raspberry Pi Zero W camera units

Note: This manual replaces the V1 laptop-controlled SSH/browser workflow as the primary method. The Windows scripts remain useful only as diagnostics or fallback.

# 1. Final V2 system

Two Raspberry Pi Zero W/WH units with Camera Module 3 Standard and 200 mm ribbon cables.

Each Pi runs an always-on Flask REST service on TCP port 8080.

The Android app checks both cameras, starts recordings concurrently, polls status, downloads both MP4 files, and opens native dual-video review.

The phone, both Pis, and any diagnostic laptop must be on the same home Wi-Fi or iPhone hotspot.

Current home network defaults: Face-On 192.168.1.119:8080; Down-the-Line 192.168.1.120:8080. These URLs remain editable in the app.

No SSH key is required by the Android app. SSH remains enabled for maintenance only.

Android phone ¦ +-- HTTP --> golfcam1 / face / port 8080 +-- HTTP --> golfcam2 / dtl / port 8080 Each Pi: rpicam-vid -> latest.h264 -> ffmpeg remux -> latest.mp4 Android: concurrent record -> status polling -> concurrent download -> native review

# 2. Hardware checklist and assembly

| Item | Quantity | Purpose |
| --- | --- | --- |
| Raspberry Pi Zero W or WH | 2 | Face-On and DTL nodes |
| Camera Module 3 Standard | 2 | Video capture |
| Pi Zero camera ribbon, 200 mm | 2 | Camera link |
| microSD card | 2 | Raspberry Pi OS |
| Pimoroni LiPo SHIM | 2 | Portable power |
| 3.7 V 1200 mAh LiPo | 2 | Portable battery |
| Tripod/enclosure | 2 | Stable positioning |
| Android phone | 1 | Control and review |
| Windows laptop | 1 | Initial setup and diagnostics |

Note: Power off each Pi before inserting or reseating the camera ribbon. The original Pi Zero W supports 2.4 GHz Wi-Fi only.

# 3. Fresh SD-card preparation (only when building from scratch)

Existing working SD cards do not need to be reflashed. For a new installation, use Raspberry Pi Imager and configure each card separately:

## 3.1 Face-On SD card

OS: Raspberry Pi OS Lite (32-bit)

Hostname: golfcam1

Username: golf

Enable SSH

Set a strong password

Set the 2.4 GHz home Wi-Fi SSID/password

Wireless LAN country: CH when used in Switzerland

## 3.2 Down-the-Line SD card

OS: Raspberry Pi OS Lite (32-bit)

Hostname: golfcam2

Username: golf

Enable SSH

Set a strong password

Set the 2.4 GHz home Wi-Fi SSID/password

Wireless LAN country: CH when used in Switzerland

# 4. Network setup on each Pi

## 4.1 Verify golfcam1

ssh golf@golfcam1.local hostname ssh golf@golfcam1.local "hostname -I" ssh golf@golfcam1.local "rpicam-vid --list-cameras"

Expected hostname: golfcam1. Camera output must include imx708.

## 4.2 Verify golfcam2

ssh golf@golfcam2.local hostname ssh golf@golfcam2.local "hostname -I" ssh golf@golfcam2.local "rpicam-vid --list-cameras"

Expected hostname: golfcam2. Camera output must include imx708.

## 4.3 Add the iPhone hotspot as a second saved network

Perform the following on golfcam1:

ssh golf@golfcam1.local sudo nmtui

Choose Edit a connection -> Add -> Wi-Fi.

Profile name: Mitul iPhone Hotspot (or another clear label).

SSID: exactly the iPhone device/hotspot name.

Security: WPA & WPA2 Personal.

Password: enter the current hotspot password.

Enable Automatically connect and Available to all users.

Save and exit.

Perform the following separately on golfcam2:

ssh golf@golfcam2.local sudo nmtui

Choose Edit a connection -> Add -> Wi-Fi.

Profile name: Mitul iPhone Hotspot (or another clear label).

SSID: exactly the iPhone device/hotspot name.

Security: WPA & WPA2 Personal.

Password: enter the current hotspot password.

Enable Automatically connect and Available to all users.

Save and exit.

Note: On iPhone, enable Personal Hotspot, Allow Others to Join, and Maximize Compatibility. Leave the hotspot page open during first pairing.

## 4.4 Set Wi-Fi priorities explicitly

First display the exact profile names on golfcam1:

ssh golf@golfcam1.local nmcli connection show

Then set hotspot priority 20 and home priority 10, replacing profile names only if your output differs:

sudo nmcli connection modify "Wi-Fi connection mituliphonehotspot" connection.autoconnect-priority 20 sudo nmcli connection modify "netplan-wlan0-FibreBox_X6-06F1A7" connection.autoconnect-priority 10 nmcli -f NAME,AUTOCONNECT,AUTOCONNECT-PRIORITY connection show exit

Now configure golfcam2 independently:

ssh golf@golfcam2.local nmcli connection show sudo nmcli connection modify "Wi-Fi connection mituliphonehotspot" connection.autoconnect-priority 20 sudo nmcli connection modify "netplan-wlan0-FibreBox_X6-06F1A7" connection.autoconnect-priority 10 nmcli -f NAME,AUTOCONNECT,AUTOCONNECT-PRIORITY connection show exit

Note: With these values: home-only -> home Wi-Fi; hotspot-only -> hotspot; both available -> hotspot. If you prefer home Wi-Fi when both exist, reverse the numbers.

# 5. Install the REST service on golfcam1 (Face-On)

ssh golf@golfcam1.local

sudo apt update sudo apt install -y python3-flask ffmpeg command -v python3 command -v ffmpeg command -v rpicam-vid rpicam-vid --list-cameras

sudo mkdir -p /opt/golfcam sudo mkdir -p /home/golf/recordings sudo chown -R golf:golf /opt/golfcam sudo chown -R golf:golf /home/golf/recordings

Create the API file:

nano /opt/golfcam/app.py

Paste the complete script below, then save with Ctrl+O, Enter, Ctrl+X.

from __future__ import annotations import os import socket import subprocess import threading import time from pathlib import Path from typing import Any from flask import Flask, jsonify, request, send_file app = Flask(__name__) HOSTNAME = socket.gethostname() CAMERA_ROLE = os.environ.get("CAMERA_ROLE", "face") PORT = int(os.environ.get("GOLFCAM_PORT", "8080")) RECORDINGS_DIRECTORY = Path("/home/golf/recordings") H264_FILE = RECORDINGS_DIRECTORY / "latest.h264" MP4_FILE = RECORDINGS_DIRECTORY / "latest.mp4" DEFAULT_DURATION_MS = 10_000 MIN_DURATION_MS = 1_000 MAX_DURATION_MS = 60_000 WIDTH = 1280 HEIGHT = 720 FPS = 30 BITRATE = 8_000_000 MINIMUM_VALID_FILE_SIZE = 100_000 state_lock = threading.Lock() camera_state: dict[str, Any] = { "state": "ready", "error": None, "recordingStartedAt": None, "recordingFinishedAt": None, } def set_state(state: str, error: str ¦ None = None) -> None: with state_lock: camera_state["state"] = state camera_state["error"] = error if state == "recording": camera_state["recordingStartedAt"] = time.time() camera_state["recordingFinishedAt"] = None if state == "ready": camera_state["recordingFinishedAt"] = time.time() def run_command(command: list[str]) -> subprocess.CompletedProcess[str]: return subprocess.run(command, check=True, capture_output=True, text=True) def recording_worker(duration_ms: int) -> None: try: set_state("recording") RECORDINGS_DIRECTORY.mkdir(parents=True, exist_ok=True) H264_FILE.unlink(missing_ok=True) MP4_FILE.unlink(missing_ok=True) subprocess.run( ["pkill", "-x", "rpicam-vid"], check=False, stdout=subprocess.DEVNULL, stderr=subprocess.DEVNULL, ) run_command([ "rpicam-vid", "-n", "-t", str(duration_ms), "--width", str(WIDTH), "--height", str(HEIGHT), "--framerate", str(FPS), "--bitrate", str(BITRATE), "-o", str(H264_FILE), ]) if not H264_FILE.exists(): raise RuntimeError("The camera did not create an H264 file.") if H264_FILE.stat().st_size < MINIMUM_VALID_FILE_SIZE: raise RuntimeError( f"The H264 file is unexpectedly small: {H264_FILE.stat().st_size} bytes." ) set_state("processing") run_command([ "ffmpeg", "-y", "-framerate", str(FPS), "-i", str(H264_FILE), "-c:v", "copy", "-movflags", "+faststart", str(MP4_FILE), ]) if not MP4_FILE.exists(): raise RuntimeError("FFmpeg did not create an MP4 file.") if MP4_FILE.stat().st_size < MINIMUM_VALID_FILE_SIZE: raise RuntimeError( f"The MP4 file is unexpectedly small: {MP4_FILE.stat().st_size} bytes." ) set_state("ready") except subprocess.CalledProcessError as error: set_state("error", error.stderr.strip() if error.stderr else str(error)) except Exception as error: set_state("error", str(error)) @app.get("/health") def health(): return jsonify({ "ok": True, "hostname": HOSTNAME, "role": CAMERA_ROLE, "apiVersion": "1.0", }) @app.get("/status") def status(): with state_lock: snapshot = dict(camera_state) mp4_exists = MP4_FILE.exists() h264_exists = H264_FILE.exists() return jsonify({ "deviceId": HOSTNAME, "hostname": HOSTNAME, "role": CAMERA_ROLE, "state": snapshot["state"], "ready": snapshot["state"] == "ready", "busy": snapshot["state"] in {"recording", "processing"}, "error": snapshot["error"], "recordingStartedAt": snapshot["recordingStartedAt"], "recordingFinishedAt": snapshot["recordingFinishedAt"], "latestAvailable": mp4_exists, "latestSizeBytes": MP4_FILE.stat().st_size if mp4_exists else 0, "rawRecordingAvailable": h264_exists, "batteryPercent": None, "apiVersion": "1.0", }) @app.post("/record") def record(): with state_lock: current_state = camera_state["state"] if current_state in {"recording", "processing"}: return jsonify({ "accepted": False, "message": "The camera is currently busy.", "state": current_state, }), 409 body = request.get_json(silent=True) or {} try: duration_ms = int(body.get("durationMs", DEFAULT_DURATION_MS)) except (TypeError, ValueError): return jsonify({ "accepted": False, "message": "durationMs must be an integer.", }), 400 duration_ms = max(MIN_DURATION_MS, min(duration_ms, MAX_DURATION_MS)) threading.Thread( target=recording_worker, args=(duration_ms,), daemon=True, ).start() return jsonify({ "accepted": True, "deviceId": HOSTNAME, "role": CAMERA_ROLE, "durationMs": duration_ms, }), 202 @app.get("/latest") def latest(): if not MP4_FILE.exists(): return jsonify({ "available": False, "message": "No completed recording is available.", }), 404 return jsonify({ "available": True, "deviceId": HOSTNAME, "role": CAMERA_ROLE, "filename": MP4_FILE.name, "sizeBytes": MP4_FILE.stat().st_size, "downloadPath": "/download/latest", }) @app.get("/download/latest") def download_latest(): if not MP4_FILE.exists(): return jsonify({ "available": False, "message": "No completed recording is available.", }), 404 return send_file( MP4_FILE, mimetype="video/mp4", as_attachment=True, download_name=f"{CAMERA_ROLE}-latest.mp4", conditional=True, ) @app.post("/reset") def reset(): with state_lock: current_state = camera_state["state"] if current_state in {"recording", "processing"}: return jsonify({ "reset": False, "message": "Cannot reset while the camera is busy.", }), 409 H264_FILE.unlink(missing_ok=True) MP4_FILE.unlink(missing_ok=True) set_state("ready") return jsonify({ "reset": True, "message": "Previous recordings were deleted.", }) if __name__ == "__main__": app.run(host="0.0.0.0", port=PORT, debug=False, threaded=True)

python3 -m py_compile /opt/golfcam/app.py

Create the Face-On environment file:

sudo nano /etc/golfcam.env

CAMERA_ROLE=face GOLFCAM_PORT=8080

Create the systemd service:

sudo nano /etc/systemd/system/golfcam.service

[Unit] Description=GolfCam Camera REST Service After=network-online.target Wants=network-online.target [Service] Type=simple User=golf Group=golf WorkingDirectory=/opt/golfcam EnvironmentFile=/etc/golfcam.env ExecStart=/usr/bin/python3 /opt/golfcam/app.py Restart=always RestartSec=3 [Install] WantedBy=multi-user.target

sudo systemctl daemon-reload sudo systemctl enable golfcam sudo systemctl start golfcam sudo systemctl status golfcam --no-pager curl http://127.0.0.1:8080/health curl http://127.0.0.1:8080/status exit

# 6. Install the REST service on golfcam2 (Down-the-Line)

ssh golf@golfcam2.local

sudo apt update sudo apt install -y python3-flask ffmpeg command -v python3 command -v ffmpeg command -v rpicam-vid rpicam-vid --list-cameras

sudo mkdir -p /opt/golfcam sudo mkdir -p /home/golf/recordings sudo chown -R golf:golf /opt/golfcam sudo chown -R golf:golf /home/golf/recordings

Create the API file:

nano /opt/golfcam/app.py

Paste the complete script below, then save with Ctrl+O, Enter, Ctrl+X.

from __future__ import annotations import os import socket import subprocess import threading import time from pathlib import Path from typing import Any from flask import Flask, jsonify, request, send_file app = Flask(__name__) HOSTNAME = socket.gethostname() CAMERA_ROLE = os.environ.get("CAMERA_ROLE", "face") PORT = int(os.environ.get("GOLFCAM_PORT", "8080")) RECORDINGS_DIRECTORY = Path("/home/golf/recordings") H264_FILE = RECORDINGS_DIRECTORY / "latest.h264" MP4_FILE = RECORDINGS_DIRECTORY / "latest.mp4" DEFAULT_DURATION_MS = 10_000 MIN_DURATION_MS = 1_000 MAX_DURATION_MS = 60_000 WIDTH = 1280 HEIGHT = 720 FPS = 30 BITRATE = 8_000_000 MINIMUM_VALID_FILE_SIZE = 100_000 state_lock = threading.Lock() camera_state: dict[str, Any] = { "state": "ready", "error": None, "recordingStartedAt": None, "recordingFinishedAt": None, } def set_state(state: str, error: str ¦ None = None) -> None: with state_lock: camera_state["state"] = state camera_state["error"] = error if state == "recording": camera_state["recordingStartedAt"] = time.time() camera_state["recordingFinishedAt"] = None if state == "ready": camera_state["recordingFinishedAt"] = time.time() def run_command(command: list[str]) -> subprocess.CompletedProcess[str]: return subprocess.run(command, check=True, capture_output=True, text=True) def recording_worker(duration_ms: int) -> None: try: set_state("recording") RECORDINGS_DIRECTORY.mkdir(parents=True, exist_ok=True) H264_FILE.unlink(missing_ok=True) MP4_FILE.unlink(missing_ok=True) subprocess.run( ["pkill", "-x", "rpicam-vid"], check=False, stdout=subprocess.DEVNULL, stderr=subprocess.DEVNULL, ) run_command([ "rpicam-vid", "-n", "-t", str(duration_ms), "--width", str(WIDTH), "--height", str(HEIGHT), "--framerate", str(FPS), "--bitrate", str(BITRATE), "-o", str(H264_FILE), ]) if not H264_FILE.exists(): raise RuntimeError("The camera did not create an H264 file.") if H264_FILE.stat().st_size < MINIMUM_VALID_FILE_SIZE: raise RuntimeError( f"The H264 file is unexpectedly small: {H264_FILE.stat().st_size} bytes." ) set_state("processing") run_command([ "ffmpeg", "-y", "-framerate", str(FPS), "-i", str(H264_FILE), "-c:v", "copy", "-movflags", "+faststart", str(MP4_FILE), ]) if not MP4_FILE.exists(): raise RuntimeError("FFmpeg did not create an MP4 file.") if MP4_FILE.stat().st_size < MINIMUM_VALID_FILE_SIZE: raise RuntimeError( f"The MP4 file is unexpectedly small: {MP4_FILE.stat().st_size} bytes." ) set_state("ready") except subprocess.CalledProcessError as error: set_state("error", error.stderr.strip() if error.stderr else str(error)) except Exception as error: set_state("error", str(error)) @app.get("/health") def health(): return jsonify({ "ok": True, "hostname": HOSTNAME, "role": CAMERA_ROLE, "apiVersion": "1.0", }) @app.get("/status") def status(): with state_lock: snapshot = dict(camera_state) mp4_exists = MP4_FILE.exists() h264_exists = H264_FILE.exists() return jsonify({ "deviceId": HOSTNAME, "hostname": HOSTNAME, "role": CAMERA_ROLE, "state": snapshot["state"], "ready": snapshot["state"] == "ready", "busy": snapshot["state"] in {"recording", "processing"}, "error": snapshot["error"], "recordingStartedAt": snapshot["recordingStartedAt"], "recordingFinishedAt": snapshot["recordingFinishedAt"], "latestAvailable": mp4_exists, "latestSizeBytes": MP4_FILE.stat().st_size if mp4_exists else 0, "rawRecordingAvailable": h264_exists, "batteryPercent": None, "apiVersion": "1.0", }) @app.post("/record") def record(): with state_lock: current_state = camera_state["state"] if current_state in {"recording", "processing"}: return jsonify({ "accepted": False, "message": "The camera is currently busy.", "state": current_state, }), 409 body = request.get_json(silent=True) or {} try: duration_ms = int(body.get("durationMs", DEFAULT_DURATION_MS)) except (TypeError, ValueError): return jsonify({ "accepted": False, "message": "durationMs must be an integer.", }), 400 duration_ms = max(MIN_DURATION_MS, min(duration_ms, MAX_DURATION_MS)) threading.Thread( target=recording_worker, args=(duration_ms,), daemon=True, ).start() return jsonify({ "accepted": True, "deviceId": HOSTNAME, "role": CAMERA_ROLE, "durationMs": duration_ms, }), 202 @app.get("/latest") def latest(): if not MP4_FILE.exists(): return jsonify({ "available": False, "message": "No completed recording is available.", }), 404 return jsonify({ "available": True, "deviceId": HOSTNAME, "role": CAMERA_ROLE, "filename": MP4_FILE.name, "sizeBytes": MP4_FILE.stat().st_size, "downloadPath": "/download/latest", }) @app.get("/download/latest") def download_latest(): if not MP4_FILE.exists(): return jsonify({ "available": False, "message": "No completed recording is available.", }), 404 return send_file( MP4_FILE, mimetype="video/mp4", as_attachment=True, download_name=f"{CAMERA_ROLE}-latest.mp4", conditional=True, ) @app.post("/reset") def reset(): with state_lock: current_state = camera_state["state"] if current_state in {"recording", "processing"}: return jsonify({ "reset": False, "message": "Cannot reset while the camera is busy.", }), 409 H264_FILE.unlink(missing_ok=True) MP4_FILE.unlink(missing_ok=True) set_state("ready") return jsonify({ "reset": True, "message": "Previous recordings were deleted.", }) if __name__ == "__main__": app.run(host="0.0.0.0", port=PORT, debug=False, threaded=True)

python3 -m py_compile /opt/golfcam/app.py

Create the Down-the-Line environment file:

sudo nano /etc/golfcam.env

CAMERA_ROLE=dtl GOLFCAM_PORT=8080

Create the systemd service:

sudo nano /etc/systemd/system/golfcam.service

[Unit] Description=GolfCam Camera REST Service After=network-online.target Wants=network-online.target [Service] Type=simple User=golf Group=golf WorkingDirectory=/opt/golfcam EnvironmentFile=/etc/golfcam.env ExecStart=/usr/bin/python3 /opt/golfcam/app.py Restart=always RestartSec=3 [Install] WantedBy=multi-user.target

sudo systemctl daemon-reload sudo systemctl enable golfcam sudo systemctl start golfcam sudo systemctl status golfcam --no-pager curl http://127.0.0.1:8080/health curl http://127.0.0.1:8080/status exit

# 7. Verify both REST APIs from Windows

curl.exe http://192.168.1.119:8080/health curl.exe http://192.168.1.119:8080/status curl.exe http://192.168.1.120:8080/health curl.exe http://192.168.1.120:8080/status

Expected roles: 192.168.1.119 -> face; 192.168.1.120 -> dtl.

## 7.1 Individual recording test: Face-On

curl.exe -X POST -H "Content-Type: application/json" -d '{\"durationMs\":3000}' http://192.168.1.119:8080/record Start-Sleep -Seconds 6 curl.exe http://192.168.1.119:8080/status curl.exe -o "$HOME\face-rest-test.mp4" http://192.168.1.119:8080/download/latest start "$HOME\face-rest-test.mp4"

## 7.2 Individual recording test: Down-the-Line

curl.exe -X POST -H "Content-Type: application/json" -d '{\"durationMs\":3000}' http://192.168.1.120:8080/record Start-Sleep -Seconds 6 curl.exe http://192.168.1.120:8080/status curl.exe --retry 3 --retry-delay 2 -o "$HOME\dtl-rest-test.mp4" http://192.168.1.120:8080/download/latest start "$HOME\dtl-rest-test.mp4"

# 8. Optional complete Windows dual-camera REST test script

Create a diagnostic script. This is not required for daily Android use, but it proves both APIs independently of the app.

notepad "$HOME\GolfCam-Dual-REST-Test.ps1"

$ErrorActionPreference = "Stop" $faceBase = "http://192.168.1.119:8080" $dtlBase = "http://192.168.1.120:8080" $durationMs = 10000 Write-Host "Checking both cameras..." $faceHealth = Invoke-RestMethod "$faceBase/health" $dtlHealth = Invoke-RestMethod "$dtlBase/health" if (-not $faceHealth.ok) { throw "Face camera health check failed." } if (-not $dtlHealth.ok) { throw "DTL camera health check failed." } Write-Host "Starting both cameras concurrently..." $faceJob = Start-Job -ScriptBlock { param($base, $duration) Invoke-RestMethod -Method Post -Uri "$base/record" -ContentType "application/json" -Body (@{durationMs=$duration} ¦ ConvertTo-Json) } -ArgumentList $faceBase, $durationMs $dtlJob = Start-Job -ScriptBlock { param($base, $duration) Invoke-RestMethod -Method Post -Uri "$base/record" -ContentType "application/json" -Body (@{durationMs=$duration} ¦ ConvertTo-Json) } -ArgumentList $dtlBase, $durationMs Wait-Job $faceJob, $dtlJob ¦ Out-Null $faceStart = Receive-Job $faceJob $dtlStart = Receive-Job $dtlJob Remove-Job $faceJob, $dtlJob if (-not $faceStart.accepted) { throw "Face camera rejected recording." } if (-not $dtlStart.accepted) { throw "DTL camera rejected recording." } Write-Host "Polling status..." $deadline = (Get-Date).AddSeconds(50) do { $face = Invoke-RestMethod "$faceBase/status" $dtl = Invoke-RestMethod "$dtlBase/status" Write-Host "Face:" $face.state "¦ DTL:" $dtl.state if ($face.state -eq "error") { throw "Face error: $($face.error)" } if ($dtl.state -eq "error") { throw "DTL error: $($dtl.error)" } if ((Get-Date) -gt $deadline) { throw "Timed out waiting for cameras." } Start-Sleep -Milliseconds 750 } until ($face.state -eq "ready" -and $dtl.state -eq "ready" -and $face.latestAvailable -and $dtl.latestAvailable) $folder = Join-Path $HOME ("GolfCam-Capture-" + (Get-Date -Format "yyyyMMdd-HHmmss")) New-Item -ItemType Directory -Path $folder ¦ Out-Null Write-Host "Downloading videos to $folder ..." Invoke-WebRequest "$faceBase/download/latest" -OutFile (Join-Path $folder "face.mp4") Invoke-WebRequest "$dtlBase/download/latest" -OutFile (Join-Path $folder "dtl.mp4") Write-Host "Complete." Start-Process (Join-Path $folder "face.mp4") Start-Process (Join-Path $folder "dtl.mp4")

Run it with:

powershell -ExecutionPolicy Bypass -File "$HOME\GolfCam-Dual-REST-Test.ps1"

# 9. Android project setup and build

Current project folder:

C:\Users\mitul\OneDrive\Training camera project\Android App\GolfCam_Android_MVP

## 9.1 Open the correct Gradle project

In Android Studio choose File -> Open.

Select GolfCam_Android_MVP itself, not its parent Android App folder.

Wait for Gradle sync.

The run configuration should use module app.

## 9.2 Build from PowerShell

cd "C:\Users\mitul\OneDrive\Training camera project\Android App\GolfCam_Android_MVP" .\gradlew.bat clean assembleDebug .\gradlew.bat testDebugUnitTest

APK output:

app\build\outputs\apk\debug\app-debug.apk

## 9.3 Install on a physical Android phone

Enable Developer Options by tapping Build number seven times.

Enable USB debugging.

Connect with a data-capable USB cable.

Accept the Allow USB debugging prompt.

$adb = "$env:LOCALAPPDATA\Android\Sdk\platform-tools\adb.exe" & $adb devices & $adb install -r ".\app\build\outputs\apk\debug\app-debug.apk"

Note: Use a physical phone for real camera testing. The emulator is appropriate for Demo mode and UI work but may not behave like a device on the physical LAN.

# 10. Configure the Android app for REST

Open GolfCam on the Android phone.

Open Settings & camera setup.

Select Real REST cameras mode.

Face-On base URL: http://192.168.1.119:8080

Down-the-Line base URL: http://192.168.1.120:8080

Tap Test REST camera for Face-On. Confirm healthy.

Tap Test REST camera for Down-the-Line. Confirm healthy.

Return to the capture screen. Both cameras should show ready.

Note: The addresses above are current home-network values. On the hotspot they may change. Use the editable URL fields until automatic service discovery is implemented.

# 11. Daily use from the Android app

Power both camera units.

Turn on the chosen network: home Wi-Fi or iPhone hotspot.

Connect the Android phone to the same network.

Wait 60-90 seconds for both Pis to boot and join Wi-Fi.

Open GolfCam.

Confirm Face-On and DTL are healthy/ready.

Position the cameras and tap Record Swing.

The app sends both POST /record requests concurrently.

Wait through Recording, Processing, and Downloading.

The native Swing Review opens with both local MP4 files.

Use Play Both, sliders, frame stepping, slow motion, sync, zoom, maximise, and impact controls.

# 12. API reference

| Method | Path | Purpose | Success |
| --- | --- | --- | --- |
| GET | /health | Basic reachability and role | 200 |
| GET | /status | ready/recording/processing/error and file state | 200 |
| POST | /record | Start asynchronous recording | 202 |
| GET | /latest | Metadata for latest MP4 | 200 or 404 |
| GET | /download/latest | Download latest MP4 | 200 or 404 |
| POST | /reset | Delete latest files when idle | 200 or 409 |

# 13. Service management and diagnostics

## 13.1 golfcam1 commands

ssh golf@golfcam1.local sudo systemctl status golfcam --no-pager sudo systemctl restart golfcam sudo journalctl -u golfcam --since "10 minutes ago" --no-pager ls -lh /home/golf/recordings/latest.h264 /home/golf/recordings/latest.mp4 ffprobe -v error -show_entries format=duration,size -of default=noprint_wrappers=1 /home/golf/recordings/latest.mp4 exit

## 13.2 golfcam2 commands

ssh golf@golfcam2.local sudo systemctl status golfcam --no-pager sudo systemctl restart golfcam sudo journalctl -u golfcam --since "10 minutes ago" --no-pager ls -lh /home/golf/recordings/latest.h264 /home/golf/recordings/latest.mp4 ffprobe -v error -show_entries format=duration,size -of default=noprint_wrappers=1 /home/golf/recordings/latest.mp4 exit

## 13.3 Confirm that the phone really contacted the Pis

The journal should show the Android phone IP making GET /health, GET /status, POST /record, repeated GET /status, and GET /download/latest requests.

# 14. Troubleshooting

App says camera unhealthy: Open each /health URL from Windows. Check that the phone and Pi are on the same network. Restart golfcam.service.

Port 8080 cannot connect: The service is absent or stopped. Run systemctl status, inspect journalctl, and verify app.py/systemd files.

Camera state becomes error: Read /status error and journal. Confirm rpicam-vid --list-cameras includes imx708 and ffmpeg exists.

Download resets: Retry. Check Pi Wi-Fi, service journal, free memory, disk space, and local curl test. The Android client should retry transient failures while closing response bodies.

Android operation failed after download: Inspect Logcat. Confirm OkHttp Response/ResponseBody objects are closed, files are saved under app-private captures, and both local URIs reach Media3.

One review panel is black: Download that Pi video to Windows. If good on Windows, inspect Android role-to-file mapping and player surface. If black on Windows, inspect camera/ribbon.

.local fails on Android: This is expected on some Android devices. Use editable numeric REST base URLs until NSD/Bonjour discovery is added.

Home and hotspot both active: The higher NetworkManager autoconnect-priority is chosen. Current recommended values prefer hotspot 20 over home 10.

No battery percentage: The Pimoroni LiPo SHIM does not provide a software fuel-gauge percentage. Do not invent a percentage.

# 15. Current V2 boundary and next work

Current: Android native control and review over local REST.

Current: home Wi-Fi and iPhone hotspot profiles saved on both Pis.

Current: manual editable base URLs; automatic discovery is not yet production-ready.

Next: Avahi _golfcam._tcp advertisement plus Android NSD and Apple Bonjour.

Next: native iOS app using the same HTTP contract.

Next: final enclosure, battery access, tripod mount, field reliability testing.

Later: authentication/pairing, production WSGI server, battery fuel gauge, custom PCB, AI analysis.

# 16. Validation checklist

[ ] Both Pis boot and connect to the selected network.

[ ] golfcam1 /health returns hostname golfcam1 and role face.

[ ] golfcam2 /health returns hostname golfcam2 and role dtl.

[ ] Both services are enabled and active after reboot.

[ ] Both cameras report imx708.

[ ] Both individual REST recording tests create valid MP4s.

[ ] The dual REST test records and downloads both views.

[ ] The Android app marks both cameras healthy.

[ ] Record Swing produces new files on both Pis.

[ ] Both videos download to the phone and open in Swing Review.

[ ] Face-On and DTL role mapping is correct.

[ ] Playback, sliders, frame controls, speed, sync, zoom and maximise work.

[ ] The system works on home Wi-Fi.

[ ] The system works on the iPhone hotspot after entering the current hotspot URLs.

# Update: RTSP Live Alignment Preview (Current Implementation)

The previous JPEG polling alignment system has been replaced.
Current alignment uses MediaMTX with low-latency RTSP/H.264.

New architecture:
Android Camera Alignment → Media3 RTSP → rtsp://<camera-ip>:8554/preview

REST API (8080) is unchanged and is still used for discovery, health, recording, downloads and status.

Additional Raspberry Pi components:

Install MediaMTX ARMv6 binary on both Pis.

Create /etc/mediamtx/mediamtx.yml.

Create mediamtx.service and enable it.

Preview URL becomes rtsp://<camera-ip>:8554/preview.

Recording service stops MediaMTX before recording and restarts it afterwards.

Android app now uses Media3 RTSP players instead of JPEG polling.

Camera Alignment releases both RTSP players before recording.

## New Ports

| Port | Purpose | Protocol |
| --- | --- | --- |
| 22 | SSH | SSH |
| 8080 | REST API | HTTP |
| 8554 | Live Preview | RTSP/H.264 |