# GolfCam V3

Quick run, health check and emergency recovery

Current architecture: Android app -> REST API -> two Raspberry Pi Zero W camera units

# A. Normal daily run - Android only

Power on golfcam1 and golfcam2.

Turn on home Wi-Fi or the iPhone hotspot.

Connect the Android phone to the same network.

Wait 60-90 seconds.

Open GolfCam.

Confirm Face-On and Down-the-Line show healthy/ready.

Tap Record Swing.

Wait for Recording -> Processing -> Downloading.

Use Swing Review. No laptop, SSH, PowerShell or browser is required.

Note: Home defaults: Face-On http://192.168.1.119:8080; DTL http://192.168.1.120:8080. Hotspot addresses may differ and can be entered in Settings.

# B. 30-second laptop health check

curl.exe http://192.168.1.119:8080/health curl.exe http://192.168.1.119:8080/status curl.exe http://192.168.1.120:8080/health curl.exe http://192.168.1.120:8080/status

Expected: golfcam1 role face, golfcam2 role dtl, both state ready.

# C. Find current IP addresses

ping golfcam1.local ping golfcam2.local

Use the addresses shown in brackets. Enter them in the app as http://<IP>:8080.

# D. Restart the Face-On REST service

ssh golf@golfcam1.local sudo systemctl restart golfcam sudo systemctl status golfcam --no-pager curl http://127.0.0.1:8080/health exit

# E. Restart the DTL REST service

ssh golf@golfcam2.local sudo systemctl restart golfcam sudo systemctl status golfcam --no-pager curl http://127.0.0.1:8080/health exit

# F. Check the latest real videos on each Pi

ssh golf@golfcam1.local "ls -lh /home/golf/recordings/latest.mp4 && ffprobe -v error -show_entries format=duration,size -of default=noprint_wrappers=1 /home/golf/recordings/latest.mp4" ssh golf@golfcam2.local "ls -lh /home/golf/recordings/latest.mp4 && ffprobe -v error -show_entries format=duration,size -of default=noprint_wrappers=1 /home/golf/recordings/latest.mp4"

# G. Download latest files to Windows

curl.exe --retry 3 --retry-delay 2 -o "$HOME\face-latest.mp4" http://192.168.1.119:8080/download/latest curl.exe --retry 3 --retry-delay 2 -o "$HOME\dtl-latest.mp4" http://192.168.1.120:8080/download/latest start "$HOME\face-latest.mp4" start "$HOME\dtl-latest.mp4"

# H. Full dual-camera diagnostic test

powershell -ExecutionPolicy Bypass -File "$HOME\GolfCam-Dual-REST-Test.ps1"

The complete setup manual contains the full contents of GolfCam-Dual-REST-Test.ps1.

# I. Build and reinstall the Android APK

cd "C:\Users\mitul\OneDrive\Training camera project\Android App\GolfCam_Android_MVP" .\gradlew.bat clean assembleDebug .\gradlew.bat testDebugUnitTest $adb = "$env:LOCALAPPDATA\Android\Sdk\platform-tools\adb.exe" & $adb devices & $adb install -r ".\app\build\outputs\apk\debug\app-debug.apk"

# J. Fast fault map

| Symptom | First action |
| --- | --- |
| Camera unhealthy | Run /health and restart that Pi service. |
| Connection refused on 8080 | Check systemctl status golfcam. |
| Recording error | Read /status and journalctl; verify imx708 and ffmpeg. |
| Download reset | Retry; restart service; check Wi-Fi. |
| One black panel | Open that Pi MP4 on Windows to separate camera vs app playback. |
| App uses old address | Update base URLs in Settings. |
| No hotspot connection | Enable Maximize Compatibility and confirm saved Wi-Fi profile. |
| No battery % | Expected with current LiPo SHIM hardware. |

# K. RTSP Live Preview

Camera alignment now uses live RTSP instead of JPEG polling.

Alignment preview: rtsp://192.168.1.119:8554/preview

DTL preview: rtsp://192.168.1.120:8554/preview

Android uses Media3 RTSP.

REST recording remains on port 8080.

If alignment works but recording reports camera busy, close the alignment screen and retry after ~1 second.

Verify MediaMTX: sudo systemctl status mediamtx --no-pager